ROS 2 Humble Installation Report

NVIDIA Jetson Orin Nano — ARM64

Wheelchair Robotics Project — Phase 1 of 6

| Field | Value |
| --- | --- |
| Report generated | 2026-05-27 10:32 CEST |
| Phase completed | System Preparation + ROS 2 base install |
| Engineer | wheelchairrob |
| Status | ✓ All verification tests passed |

# 1. System Identification

| Attribute | Value |
| --- | --- |
| Hostname | ubuntu |
| User | wheelchairrob |
| Hardware | NVIDIA Jetson Orin Nano (8GB) |
| CPU | ARM Cortex-A78AE, 6 cores @ aarch64 |
| RAM (physical) | 7.4 GiB |
| Storage | NVMe SSD /dev/nvme0n1p1 — 233 GB total, 185 GB free |
| OS | Ubuntu 22.04.5 LTS (Jammy) |
| Kernel | 5.15.185-tegra |
| JetPack version | 6.2.2+b24 |
| L4T (Tegra) | R36 Revision 5.0 (2026-01-16) |
| CUDA toolkit | 12.6.68 (V12.6.r12.6) |

# 2. Pre-flight Configuration Applied

## 2.1 Swap configuration

A 16 GiB swap file was created on the NVMe SSD at /mnt/swapfile, persisted in /etc/fstab for automatic mount on boot.

Rationale: The Orin Nano 8GB has insufficient RAM for full ROS 2 + Nav2 + ZED workspace compilation. Placing swap on NVMe (not eMMC/SD) avoids flash wear and provides acceptable throughput during builds.

## 2.2 ZRAM disabled

The nvzramconfig service was disabled to prevent competition with NVMe-based swap.

Verification: systemctl is-enabled nvzramconfig returns disabled.

## 2.3 Boot target

| State | Value | Action |
| --- | --- | --- |
| Current | graphical.target (GUI active) | Switch to multi-user before heavy builds |
| Recommended for builds | multi-user.target | Frees ~1 GB RAM for colcon |

## 2.4 Power mode

⚠ Action required: Currently in 15W mode. Reapply MAXN before next session:

sudo nvpmodel -m 0

sudo jetson_clocks

sudo nvpmodel -q   # confirm MAXN

# 3. ROS 2 Humble Installation

## 3.1 Repository configuration

ROS 2 official apt repository added with proper GPG signing. Locale set to en_US.UTF-8 (required by ROS 2).

## 3.2 ROS 2 packages installed

| Package | Version | Purpose |
| --- | --- | --- |
| ros-humble-ros-base | 0.10.0-1jammy.20260426.092717 | Core ROS 2 Humble runtime |
| ros-dev-tools | 1.0.1 | Developer toolkit (colcon, rosdep, vcstool) |

Note: The full desktop variant (ros-desktop-full) was intentionally not installed to save RAM and disk space. RViz2 and similar GUI tools can be added on demand.

## 3.3 Build toolchain

| Tool | Version | Notes |
| --- | --- | --- |
| GCC / G++ | 11.4.0 | C++17/C++20 capable |
| CMake | 3.22.1 | Stock Jammy version, sufficient for Humble |
| Python | 3.10.12 | Required by ROS 2 Humble |

## 3.4 Colcon plugin status

| Plugin | Version | Status |
| --- | --- | --- |
| colcon-core | 0.20.1 | ✓ up-to-date |
| colcon-cmake | 0.2.29 | ✓ up-to-date |
| colcon-ros | 0.5.0 | ✓ up-to-date |
| colcon-clean | 0.2.1 | ✓ up-to-date |
| colcon-mixin | 0.2.3 | ✓ up-to-date |

## 3.5 Rosdep & shell integration

- Rosdep initialized at /etc/ros/rosdep/sources.list.d/20-default.list

- User cache populated in ~/.ros/rosdep/

- rosdep update executed as user (correct — not as root)

- source /opt/ros/humble/setup.bash appended to ~/.bashrc

- ROS_DISTRO resolves to humble

# 4. Verification Tests

| Test | Result |
| --- | --- |
| ros2 --help returns full subcommand list | ✓ Pass |
| g++ --version reports 11.4.0 | ✓ Pass |
| cmake --version reports 3.22.1 | ✓ Pass |
| colcon version-check: all plugins healthy | ✓ Pass |
| ros2 pkg prefix rclcpp returns /opt/ros/humble | ✓ Pass |
| rclcpp C++ headers present and complete | ✓ Pass |

# 5. Resource Snapshot at Report Time

| Resource | Used | Available | Total |
| --- | --- | --- | --- |
| RAM | 2.5 GiB | 4.8 GiB | 7.4 GiB |
| Swap | 0 B | 15 GiB | 15 GiB |
| Root filesystem | 36 GB | 185 GB | 233 GB |

# 6. Pending Phases

| Phase | Status | Notes |
| --- | --- | --- |
| 1. ROS 2 Humble base install | ✓ Complete | Verified working |
| 2. ZED SDK 5.3 + zed-ros2-wrapper | → Next | Use L4T 36.5 build, CUDA 12.6 |
| 3. RoboSense rslidar_sdk | Pending | Requires 192.168.1.x Ethernet config |
| 4. Nav2 + robot_localization + PCL | Pending | Mostly apt binaries on ARM64 |
| 5. Safe workspace compilation | Pending | MAKEFLAGS=-j2 --executor sequential |
| 6. Sensor verification | Pending | Topic rates, TF tree, RViz2 |

# 7. Operational Notes & Reversion Commands

## Boot target management

# Disable GUI at boot (saves ~1 GB RAM)

sudo systemctl set-default multi-user.target

# Re-enable GUI at boot

sudo systemctl set-default graphical.target

# Temporarily kill GUI for one session (no reboot needed)

sudo systemctl isolate multi-user.target

## Monitoring during builds

sudo tegrastats        # real-time RAM, GPU, thermals

watch -n 1 free -h     # RAM usage with refresh

htop                   # process-level view

## Future: Network reservation for RoboSense LiDAR

The RS-Helios-16P will require eth0 statically configured at 192.168.1.102/24 (UDP ports 6699 MSOP / 7788 DIFOP). When that phase begins, either free eth0 from current use or add a USB-Ethernet adapter (Realtek RTL8153 or ASIX AX88179 chipsets recommended) for non-LiDAR traffic.

# 8. Replication Quick-Reference

Complete command sequence to replicate this exact installation on another Jetson Orin Nano with JetPack 6.2.2:

### Step 1: Pre-flight

sudo nvpmodel -m 0 && sudo jetson_clocks

sudo fallocate -l 16G /mnt/swapfile

sudo chmod 600 /mnt/swapfile

sudo mkswap /mnt/swapfile

sudo swapon /mnt/swapfile

echo '/mnt/swapfile none swap sw 0 0' | sudo tee -a /etc/fstab

sudo systemctl disable nvzramconfig

### Step 2: Locale and repository

sudo apt update && sudo apt install -y locales

sudo locale-gen en_US en_US.UTF-8

sudo update-locale LC_ALL=en_US.UTF-8 LANG=en_US.UTF-8

sudo apt install -y software-properties-common curl gnupg lsb-release

sudo add-apt-repository universe

sudo curl -sSL https://raw.githubusercontent.com/ros/rosdistro/master/ros.key \

-o /usr/share/keyrings/ros-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/ros-archive-keyring.gpg] \

http://packages.ros.org/ros2/ubuntu $(. /etc/os-release && echo $UBUNTU_CODENAME) main" \

| sudo tee /etc/apt/sources.list.d/ros2.list > /dev/null

### Step 3: Install packages

sudo apt update && sudo apt upgrade -y

sudo apt install -y ros-humble-ros-base ros-dev-tools \

python3-colcon-common-extensions python3-rosdep python3-vcstool \

python3-colcon-mixin build-essential cmake git

### Step 4: Rosdep and shell

sudo rosdep init

rosdep update      # WITHOUT sudo — critical

echo "source /opt/ros/humble/setup.bash" >> ~/.bashrc

source ~/.bashrc

End of report — next phase: ZED SDK 5.3 installation for L4T 36.5